BARNABUS - Extracting AMM15 modelled data from a single point:

The data source is Copernicus found at:

http://marine.copernicus.eu/

- Click on service portfolio

- Scroll down to NORTHWESTSHELF_ANALYSIS_FORECAST_PHYS_004_013

- Click Add to cart

- Click Data download

- Feel free to use my account – or make your own. Its free;

USERNAME: jduell1

PASSWORD: Geophysics2015!

There’s a list of variables available; your looking at ocean bottom temperatures (OBT) and sea surface temperatures (SST) which correspond to:

“METO-NWS-PHY-DM-BED” is ocean bottom temperature (OBT). Met office North West European Shelf model Physical Daily mean bed FYI.

“METO-NWS-PHY-HI-TEM” is Sea Water Potential Temperature which with the correct boxes ticked becomes (SST)

- For now choose TEM

For an example (E,N) coordinate:

e.g.  (-1.838364 E, 49.051234 N)

- Round to the nearest 1, and take a couple of degrees either side to make NESW bounds:

Rounded coordinate (-2 E, 49N)

NESW Boundaries:  N(50) , E(0), W(-4), S(47)

- Enter these into the selection window

- Make the time range start at the earliest possible and finish on the latest possible (take note of these date/times)

- Check the start depth is -1m and end depth is 1m making (SPT equivalent to SST)

- Select download

This is the user manual for the dataset – may be worth giving it a skim at some point but its not important for this process.

http://resources.marine.copernicus.eu/documents/QUID/CMEMS-NWS-QUID-004-013.pdf

- This is the NetCDF

Now we are working in arcMap

NB: Just change the coordinates – making this as I’m working on a project ….

- In the toolbox search “make netCDF table view”

- Add the netCDF

- Add “bottom to Variables”

- Call the output table what you like

- Make Row Dimensions “time”

- Make Dimension Values “lon” and “lat” and make the values the coordinate you are interested in

Run the function

- RHC the table generated in arcMap and press Open

- Export the table as a .txt